Error Detection in serial Data

The checksum method is a very simple error-detection technique.

- You send all data bytes + one extra checksum byte.

- The receiver adds them all (dropping carry, 8-bit sum).

- If the result is 0x00, the data is assumed good.

- If not, there’s an error in transmission/storage.

When serial send data is sent, each data being written down in hexadecimal. At the end, calculate a special number (checksum) by taking the 2’s complement of the total. When the receiver adds up all the values including the checksum, if the result is zero, the data is intact. If not zero, an error has occurred.

Check sum calculation:

#include<p18f452.h>

void main(void) {

unsigned char mydata[4] = {0x25, 0x62, 0x3F, 0x52};

unsigned char checksum = 0;

unsigned char i;

unsigned int sum = 0;

TRISC = 0x00; 						// PORTC as output (to show data)

TRISB = 0x00; 						// PORTB as output (to show checksum)

// Step 1: Add all data bytes

for(i = 0; i < 4; i++) {

PORTC = mydata[i];   					// show each byte on PORTC

sum += mydata[i];

}

// Step 2: Take 2’s complement of sum

checksum = (unsigned char)(~sum + 1);

// Step 3: Output checksum on PORTB

PORTB = checksum;

}

Check sum verification:

#include<p18f452.h>

void main(void) {

unsigned char mydata[4] = {0x25, 0x62, 0x3F, 0x52};

unsigned char checksum = 0xE8; 				// From previous calculation

unsigned char i;

unsigned int sum = 0;

TRISD =0; 						// PORTD output (to show result)

// Step 1: Add all data + checksum

for(i = 0; i < 4; i++) {

sum += mydata[i];

}

sum += checksum;

// Step 2: Check if result == 0

if((unsigned char)sum == 0) {

PORTD = 'G';   					// Send ASCII 'G' → GOOD

} else {

PORTD = 'B';   					// Send ASCII 'B' → BAD

}

}